Belt Remaining Useful Life (RUL) Prediction Model

Final Project Report (Company Live Project)

Organization: BEUMER Group
Department: Digitalization
Project Type: Company Live Project

Role (Coding & Development): Mandeep Rana
Data Extraction & Management: Sarvan
Project Review & Technical Guidance: Kannan Sir, Ganesh Sir

Project Version: 1.0
Date: December 18, 2025

1. Executive Summary

This report presents the design, development, and final assessment of the Belt Remaining Useful Life (RUL) Prediction Model, developed as a live industrial project at BEUMER Group under the Digitalization Department.

The project focuses on predicting the degradation and remaining service life of BEUMER HGBW-HC 400 x 29100 bucket elevator steel cord belts using a hybrid approach that combines machine learning models with physics-based engineering rules.

The implemented system enables predictive maintenance, reduces unplanned downtime, and improves operational safety by integrating a safety override mechanism, where real-time sensor alarms take priority over algorithmic predictions.

The final model achieved a Health Score of 95.32%, with a predicted Remaining Useful Life (RUL) of 2009 days, and an accuracy (R² score) greater than 0.99, validating its reliability for industrial deployment.

2. Project Background & Scope

2.1 Industry Context

Conveyor and bucket elevator belts are critical components in bulk material handling systems. Unexpected belt failure can lead to severe production loss and safety hazards. Traditional maintenance approaches are either reactive or time-based, which are inefficient and costly.

This project was initiated to introduce a data-driven predictive maintenance solution within BEUMER Group’s digitalization framework.

2.2 Target Application

- Equipment: BEUMER HGBW-HC 400 × 29100 Bucket Elevators

- Client Site: J.K. Cement Limited, Aligarh

- Belt Type: Steel Cord Belt (BEUMER Extra Series)

- Belt Width: 400 mm

- Belt Length: ~58.2 m

- Designed Life: 60 Months (5 Years)

2.3 Project Objectives

- Predict belt degradation and remaining useful life accurately

- Enable proactive and condition-based maintenance

- Integrate safety logic using real sensor flags

- Optimize belt life and reduce maintenance costs

3. System Architecture & Technical Implementation

3.1 Overall Architecture

graph TD

A[Raw Sensor Data] --> B[Data Preprocessing & Cleaning]

B --> C[Flag Generation Logic]

C --> D[Feature Engineering]

D --> E[Machine Learning Model (Random Forest)]

C --> F[Safety Override System]

E --> G[Health Score & RUL Prediction]

F --> H[Final Alert & Maintenance Recommendation]

G --> H

3.2 Methodology

The prediction engine is built around a Synthetic Health Score, representing the belt’s condition on a 0–100 scale. This score is derived from time-based degradation and operational stress factors.

3.3 Health Score Calculation

1. Base Health (Time-Based Decay)

Accounts for natural wear over time, including a 20% design life buffer.

[
\text{Base Health} = 100 - \left(\frac{\text{Current Age}}{\text{Baseline Life} \times 1.2}\right) \times 100
]

2. Stress-Based Penalties

Additional penalties are applied based on real sensor data:

- Elongation Penalty:
Proportional deduction (up to 40 points) based on belt stretch vs. critical limit

- Temperature Penalty:
Thermal stress deduction (up to 20 points)

[
\text{Health Score} = \text{Base Health} - \text{Total Stress Penalty}
]

3.4 Remaining Useful Life (RUL) Calculation

A power-law degradation model is used to reflect accelerated wear as health decreases:

[
\text{RUL (Days)} = \left(\frac{\text{Health Score}}{100}\right)^{1.5} \times \text{Baseline Life}
]

4. Flag Logic & Safety Override Mechanism

To ensure industrial safety, the system prioritizes physical sensor alarms over machine learning predictions.

4.1 Flag Interpretation Logic

| Sensor Type | Configured Limit | Logic Condition | Meaning |
| --- | --- | --- | --- |
| Standard Sensor | > 0 | Value ≥ Threshold | Limit exceeded |
| Binary Sensor | 0 | Value = 0 | Fault condition |

4.2 Alert Priority Order

- Critical Sensor Flag → Immediate shutdown / action

- Warning Sensor Flag → Planned maintenance required

- Health Score < 55 → Predicted critical condition

- Normal → System healthy

This layered approach ensures fail-safe operation.

5. Model Assessment & Results

Evaluation Date: December 17, 2025

5.1 Key Performance Indicators

| Parameter | Value | Status |
| --- | --- | --- |
| Health Score | 95.32% | Excellent |
| Remaining Useful Life | 2009 Days | High Confidence |
| Operating Hours | 1104.0 Hrs | — |
| Health Model R² | 0.997 | Very High Accuracy |
| RUL Model R² | 0.997 | Very High Accuracy |

5.2 Current System Recommendation

System Status: NORMAL
All sensor values are within permissible limits. No warning or critical flags detected.

6. Deployment Status & Future Enhancements

6.1 Current Deployment

- Programming Language: Python 3.8+

- Execution: Local automated pipeline

- Main Script: run_pipeline.py

- Configuration: Centralized JSON-based threshold and parameter management

6.2 Future Enhancements

- Integration of LSTM / Deep Learning models for time-series anomaly detection

- Docker containerization for scalable deployment

- Real-time integration with SCADA & dashboards

- Fleet-level belt health monitoring across multiple plants

7. Conclusion

The Belt RUL Prediction Model successfully demonstrates how machine learning combined with engineering domain knowledge can significantly improve maintenance strategies in industrial environments.

The model delivers:

- High prediction accuracy (R² > 0.99)

- Strong safety assurance through override logic

- Explainable and reliable health metrics

As a company live project at BEUMER Group, this work contributes directly to digital transformation initiatives by enabling predictive, safe, and cost-effective belt maintenance.

8. Acknowledgement

I sincerely thank BEUMER Group for providing the opportunity to work on this live industrial project.
Special thanks to Kannan Sir and Ganesh Sir for their valuable guidance and technical reviews, and to Sarvan for support in data extraction and management within the Digitalization Department.